Gamma Correction and Tone Processing Study Notes

1. Why this topic matters
Human vision is non-linear. Gamma and tone processing align image values with perception.

2. Power Law Transformations
s = c * r^gamma for r in [0,1]. gamma<1 brightens, gamma>1 darkens.
Worked example: r=0.25, gamma=0.5 gives 0.5. r=0.25, gamma=2.2 gives about 0.047.

3. Perceptual motivation
Non-linear transfer allocates precision where perception is strongest.

4. Gamma Encoding and Decoding
Encode linear values for display, decode encoded values for linear operations.

5. Non-linear standards
sRGB piecewise transfer, Log encodings, Rec.2020 context.

6. Tone Curves
Sigmoid/S-curves shape midtone contrast and protect extremes.

7. HDR Tone Mapping
Global and local operators compress scene range to display range.

8. Real-world applications
Photography, cinema post, medical imaging, machine vision preprocessing.

9. Common misconceptions
Gamma is not simple brightness, and tone mapping does not recover clipped data.

10. Key papers and standards
[1] Reinhard et al. SIGGRAPH 2002. URL: https://dl.acm.org/doi/10.1145/566570.566575
[2] Mantiuk et al. ACM TOG 2006. URL: https://dl.acm.org/doi/10.1145/1141911.1141915
[3] IEC 61966-2-1 sRGB. URL: https://www.color.org/chardata/rgb/srgb.xalter
[4] ITU-R BT.2020. URL: https://www.itu.int/rec/R-REC-BT.2020